CompTia A+

A+ - Peripherals and Power Supply

Assignment-4

Day 4: Connecting and Testing Peripherals Practical (1 hour): Set up and test peripherals like printers and webcams. Task: Resolve connectivity issues for a given setup.

QUE-01 Connect a USB webcam to your computer, install any required drivers, and verify that the webcam works by opening the default camera app and capturing a photo.

ANS

Connect and Test a USB Webcam

Step 1: Connect the webcam

- Plug the USB webcam into an available USB port on your computer.

- Wait for Windows to detect the device.

Step 2: Install the driver

- Windows may automatically install the required driver.

- If it doesn't, open Device Manager by right-clicking the Start button.

- Look under Cameras or Imaging devices.

- If the webcam has a warning symbol, right-click it and select Update driver.

Step 3: Open the Camera app

- Click the Windows Start menu.

- Search for Camera.

- Open the Camera app.

- If prompted, allow camera access in Windows settings.

Step 4: Test the webcam

- Make sure the webcam is selected if multiple cameras are available.

- Check that your live video appears on the screen.

- Click the Take Photo button (camera icon).

- Open the saved photo to confirm that the webcam works correctly.

Result

The USB webcam was connected successfully, the necessary driver was installed, and the webcam was tested by capturing a photo using the Windows Camera app.

QUE-02 Set up a local printer (USB or network), install the correct driver, and print a test page to confirm successful installation.

ANS

Set Up a Local Printer and Print a Test Page

Follow these steps on your Windows computer.

Step 1: Connect the printer

- Turn on the printer.

- USB printer: Connect the printer to your computer using a USB cable.

- Network printer: Connect the printer and computer to the same Wi-Fi network.

Step 2: Install the printer driver

- Open Settings on your computer.

- Go to Bluetooth & devices → Printers & scanners in Windows 11. In Windows 10, go to Devices → Printers & scanners.

- Click Add device.

- Select your printer and click Add device.

- If Windows cannot find the correct driver, download it from the printer manufacturer's official website using your printer's model number.

Step 3: Print a test page

- Open Settings → Printers & scanners.

- Select your installed printer.

- Open Printer properties. You may need to select Print test page under the printer's properties.

- Click Print Test Page.

- Check that the page prints correctly without errors.

Step 4: Verify installation

- Confirm that the printer appears in the list of installed printers.

- Check that the printed test page contains the Windows printer test information.

- Ensure the printer is not showing an Offline or Error status.

Result

The printer was connected successfully, the correct driver was installed, and a test page was printed to verify that the printer was working properly.

QUE-03 Simulate a connectivity issue by unplugging the printer's USB cable, then use Windows Device Manager to identify and resolve the 'printer not detected' problem.<br><br><em><strong>Hint:</strong> Check for error icons and use the 'Scan for hardware changes' option after reconnecting.</em>

ANS

Troubleshoot a Printer Not Detected Issue Using Device Manager

Follow these steps on your Windows computer.

Step 1: Simulate the connectivity issue

- Turn on the printer.

- Unplug the printer's USB cable from the computer.

- Observe that the printer may become unavailable or stop responding.

Step 2: Open Device Manager

- Right-click the Start button.

- Select Device Manager.

- Expand Printers, Print queues, and Universal Serial Bus controllers.

- Look for a printer entry with a yellow warning icon or an unknown device. The printer may also disappear from the list when disconnected.

Step 3: Reconnect the printer

- Plug the USB cable back into the computer.

- Make sure the printer is turned on.

- In Device Manager, click the Action menu.

- Select Scan for hardware changes.

- Wait for Windows to detect the printer again.

Step 4: Resolve the issue

- If the printer appears with a yellow warning icon, right-click it and select Update driver.

- Choose Search automatically for drivers.

- If the issue persists, try another USB port or cable.

- Check Settings → Printers & scanners to confirm that the printer is listed.

Step 5: Verify the solution

- Confirm that the printer appears in Device Manager without a warning icon.

- Open a document and print a test page to verify that the printer works.

Result: The printer connectivity issue was simulated by disconnecting the USB cable. After reconnecting the cable and scanning for hardware changes, the printer was detected and its functionality was verified.

QUE-04 A user reports that their webcam video is not displaying in Zoom, but it works in the default camera app. Troubleshoot and resolve the issue so the webcam works in both applications.<br><br><em><strong>Hint:</strong> Check if another app is using the webcam or if Zoom permissions are enabled.</em>

ANS

Troubleshoot Webcam Not Working in Zoom

Problem: The webcam works in the default Camera app but does not display video in Zoom.

Step 1: Close other applications

- Close the Windows Camera app and any other application using the webcam.

- Close and reopen Zoom.

- Join a test meeting and check whether the video appears.

Step 2: Check camera permissions

- Open Settings → Privacy & security → Camera in Windows 11. In Windows 10, open Settings → Privacy → Camera.

- Turn on Camera access.

- Enable Let desktop apps access your camera.

- Ensure Zoom has permission to access the webcam.

Step 3: Select the correct camera in Zoom

- Open Zoom.

- Click your profile picture and select Settings.

- Select Video & effects or Video.

- Under Camera, select the correct webcam from the list.

- Check whether the video preview appears.

Step 4: Restart and update Zoom

- Exit Zoom completely.

- Reopen Zoom and test the webcam.

- If the problem continues, update Zoom to the latest version.

Step 5: Verify the solution

- Open the Windows Camera app and confirm that the webcam works.

- Open Zoom and check the video preview.

- Join a test meeting to confirm that the webcam displays video in both applications.

Result: The webcam issue can be resolved by closing applications that may be using the camera, enabling the required permissions, and selecting the correct webcam in Zoom. Verify the result in both applications.